# AMC 1968-80 VIN (serial number) decoder

Examples:

| A | 2 | C | 7 | 9 | 8 | Z | 1 | 23456 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- A = AMC

- 2 = Year (1972)

- C = transmission type (3 speed auto on floor)

- 7 = car series (Javelin)

- 9 = body type (2 door hardtop)

- 8 = body class (Javelin AMX)

- Z = engine (401-4v)

- 1 = assembly location (Kenosha)

- 23456 = production sequence number

| A | 4 | C | 7 | 9 | 7 | H | 1 | 97173 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- A = AMC

- 4 = Year (1974)

- C = transmission type (3 speed auto on floor)

- 7 = car series (Javelin)

- 9 = body type (2 door hardtop)

- 7 = body class (Javelin Javelin)

- H = engine (304-2v)

- 1 = assembly location (Kenosha)

- 97173 = production sequence number

| Manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| letter | 1968 | 1969 | 1970 | 1971 | 1972 | 1973 | 1974 | 1975 | 1976 | 1977 | 1978 | 1979 | 1980 |
| A | American Motors Corporation |  |  |  |  |  |  |  |  |  |  |  |  |
| other | brand X |  |  |  |  |  |  |  |  |  |  |  |  |
| Year |  |  |  |  |  |  |  |  |  |  |  |  |  |
| number | 1968 | 1969 | 1970 | 1971 | 1972 | 1973 | 1974 | 1975 | 1976 | 1977 | 1978 | 1979 | 1980 |
| 0 |  |  | 1970 |  |  |  |  |  |  |  |  |  | 1980 |
| 1 |  |  |  | 1971 |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  | 1972 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | 1973 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  | 1974 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  | 1975 |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  | 1976 |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  | 1977 |  |  |  |
| 8 | 1968 |  |  |  |  |  |  |  |  |  | 1978 |  |  |
| 9 |  | 1969 |  |  |  |  |  |  |  |  |  | 1979 |  |
| Transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| letter | 1968 | 1969 | 1970 | 1971 | 1972 | 1973 | 1974 | 1975 | 1976 | 1977 | 1978 | 1979 | 1980 |
| A | 3spd/auto/ column |  |  |  |  |  |  |  |  |  |  |  |  |
| C | 3spd/auto/ floor | 3spd/auto/ console | 3spd/auto/ floor |  |  |  |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  | 3spd/man/ floor overdrive |  |  |  |  |  |
| E |  |  |  |  | 3spd/man/ floor full-synchro | 3spd/man/ floor |  |  |  |  |  |  |  |
| F | 4spd/man/ console |  |  | 3spd/man/ floor | 3spd/man/ floor 2-3 synchro | 3spd/man/ floor |  |  |  |  |  |  |  |
| M | 4spd/man/ floor |  |  |  |  |  |  |  |  | 4spd/man/ floor |  |  |  |
| O | 3spdOD/ man/ column |  |  |  |  |  |  | 3spdOD/ man/ column |  |  |  |  |  |
| S | 3spd/man/ column | 3spd/man/ column (or 3spd/man/ floor, Javelin) | 3spd/man/ column or floor | 3spd/man/ column |  |  |  |  |  |  |  |  |  |
| Car Series |  |  |  |  |  |  |  |  |  |  |  |  |  |
| number | 1968 | 1969 | 1970 | 1971 | 1972 | 1973 | 1974 | 1975 | 1976 | 1977 | 1978 | 1979 | 1980 |
| 0 | American | Rambler | Hornet |  |  |  |  |  |  |  | Concord |  |  |
| 1 | Rebel |  |  | Matador 2 Door |  |  |  |  |  |  |  |  |  |
| 3 | AMX |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | Gremlin |  |  |  |  |  |  |  |  | Spirit |  |
| 6 |  |  |  |  |  |  |  | Pacer |  |  |  |  |  |
| 7 | Javelin |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | Ambassador |  |  |  |  |  |  | Matador 4 Door/Wagon |  |  |  |  |  |
| ? |  |  |  |  |  |  |  |  |  |  |  |  | Eagle |
| Body Type |  |  |  |  |  |  |  |  |  |  |  |  |  |
| number | 1968 | 1969 | 1970 | 1971 | 1972 | 1973 | 1974 | 1975 | 1976 | 1977 | 1978 | 1979 | 1980 |
| 3 |  |  |  |  |  | 2 door hatchback |  |  |  |  |  |  |  |
| 5 | 4 door sedan |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | 2 door sedan |  |  |  |  |  | 2 door sedan (coupe) |  |  |  |  |  |  |
| 7 | 2 door convertible |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 4 door wagon |  |  |  |  |  |  |  |  | station wagon |  |  |  |
| 9 | 2 door hardtop |  |  |  |  |  |  |  |  |  |  |  |  |
| Group (Body Class) (Model) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| number | 1968 | 1969 | 1970 | 1971 | 1972 | 1973 | 1974 | 1975 | 1976 | 1977 | 1978 | 1979 | 1980 |
| 0 | American, 550 | Rambler, Rebel | Hornet, Rebel | Gremlin 2 seat, Hornet |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  | Hornet SC360 |  |  |  |  |  |  |  |  |  |
| 2 | Ambassador |  |  | Ambassador DPL |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  | Gremlin |  |  |  |
| 5 | 440, 770, DPL | 440, Javelin, DPL | Javelin, DPL | Gremlin 4 seat, Javelin, Ambassador SST | Gremlin, Ambassador SST | Gremlin |  |  |  |  |  |  |  |
| 7 | AMX, Rogue, SST |  |  | Hornet SST, Hornet wagon, Javelin SST, Matador, Ambassador Brougham. | Hornet SST, Hornet wagon, Javelin SST, Matador, Ambassador Brougham. | Ambassador, Hornet, Javelin, Matador, |  | Hornet, Matador, Pacer |  |  | Concord, Gremlin, Matador, Pacer |  |  |
| 8 |  |  |  | Javelin AMX |  |  |  | Javelin AMX, Matador X |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  | AMX |  |  |
| P |  |  |  |  |  |  | Police |  |  |  |  |  |  |
| T |  |  |  |  |  |  | Taxi |  |  |  |  |  |  |
| Engine |  |  |  |  |  |  |  |  |  |  |  |  |  |
| letter | 1968 | 1969 | 1970 | 1971 | 1972 | 1973 | 1974 | 1975 | 1976 | 1977 | 1978 | 1979 | 1980 |
| A | 199-1v |  |  | 258-1v |  |  |  |  |  |  |  |  |  |
| B | 232-1v |  |  |  |  | 258-1v L/C |  |  |  |  |  |  |  |
| C | 232-2v |  |  |  |  |  |  |  | 258-2v |  |  |  |  |
| E |  |  |  | 232-1v |  |  |  |  |  |  |  |  |  |
| G |  |  |  |  |  |  |  |  |  | 121-2v |  |  |  |
| H |  |  | 304-2v |  |  |  |  |  |  |  |  |  |  |
| K |  |  |  |  |  |  |  |  |  | 121-2v |  |  |  |
| M | 290-2v |  | 304-4v |  | ? | 304-4v |  |  |  |  |  |  |  |
| N | 290-4v |  | 360-2v |  |  |  |  |  |  |  |  |  |  |
| P |  |  | 360-4v |  |  |  |  |  |  |  |  |  |  |
| S | 343-2v |  | 390-2v |  |  |  |  |  |  |  |  |  |  |
| T | 343-4v |  |  |  |  |  |  |  |  |  |  |  |  |
| W | 390-2v |  |  |  |  |  |  |  |  |  |  |  |  |
| X | 390-4v |  |  |  |  |  |  |  |  |  |  |  |  |
| Y |  |  | 390-4v "Machine"* |  |  |  |  |  |  |  |  |  |  |
| Z |  |  |  | 401-4v |  |  |  |  |  |  |  |  |  |
| Assembly Location |  |  |  |  |  |  |  |  |  |  |  |  |  |
| number | 1968 | 1969 | 1970 | 1971 | 1972 | 1973 | 1974 | 1975 | 1976 | 1977 | 1978 | 1979 | 1980 |
| 0 |  |  |  |  |  | Kenosha w/REABS |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  | Kenosha |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  | Brampton |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  | Brampton w/REABS |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  | Brampton |  |  |  |  |  |  |  |
| K |  |  |  |  |  | kit (export) |  |  |  |  |  |  |  |

* The Rebel "Machine" engine used a unique intake and exhaust.